# UROVO

# CT58S

Specifications

UROVO Technology Co., Ltd.

Specifications

| Performance | Model | UROVO CT58S |
| --- | --- | --- |
|  | O.S. | Android 12.0 |
|  | Processor | Octa-core 2.0GHz |
|  | Memory | RAM: 4GB , ROM: 64GB |
|  | Extended memory | Micro SD card, Up to 256 GB SDXC |
| Basic specifications | Dimensions | 156.9*72.3*16.9mm |
|  | Weight | 9.38 oz./266 g with standard battery |
|  | Display | 5.5 inch display, 720*1440 |
|  | Touch Screen | Ultra sensitive capacitive touch panel, support multi-touch, works with gloves and wet fingers |
|  | Main battery | Capacity：3.85V 5000mAh |
|  | Charging time | Less than 4 hours |
|  | Buttons | PWR button, Vol+/- button, Scan button*2 |
|  | Camera | Front 5MP, Fixed Focus Rear 13MP with flash, Auto Focus |
|  | Sensors | Light + Accelerator+ Proximity |
|  | Scanning | Professional scan engine Support international standard 1D/2D barcode； Support barcode displayed on screen and colored barcode； |
|  | Slots | One slot, Nano-SIM x 1(Either Micro SD x 1 up to 256GB), eSIM x 1(optional) |
|  | Audio | Single-Microphone with noise cancellation, 1.5W Speaker for loud noise , earpiece supports echo cancellation |
|  | Interfaces | USB Type-C、pogo pin |
| Network Connections | Positioning | GPS、A-GPS、BEIDOU、GLONASS、Galileo |
|  | WWAN | 4G/3G/2G |
|  | Bluetooth | BT5.0 + BR/EDR + BLE |
|  | Wi-Fi | 2.4G/5G，IEEE 802.11a/b/g/n/ac/d/e/h/i/k/r/v/w Roaming：802.11r /OKC/ PMKID caching |
| Environment | Operating Temp. | -20°C to 60°C |
|  | Storage Temp. | -40°C to 70°C |
|  | Humidity | 5%RH～95%RH (No condensation) |
|  | Drop Specification | Multiple 1.5 m drops to concrete at room temperature |
|  | Tumbling | 400 1.0m tumbles |
|  | Sealing | IP67 |
|  | ESD | +/-15kv Air; +/-8kv contact |

Data Capture Specifications

  - Camera

| Front Camera |  |
| --- | --- |
| Pixels | 5MP |
| Rear Camera |  |
| Functions | Flash light(700mA), Auto-focus, 1080p video recording |
| Pixels | 13MP, Auto-focus, 1080p video recording |

  - Scan Engine

|  | 2D Imager Engine Specifications |
| --- | --- |
| Scan Angle Tilt | 360° |
| Scan Angle Pitch | ±60° |
| Scan Angle Skew | ±60° |
| Barcodes Support | Linear: Codabar, Code 11, Code 128, Code 2 of 5, Code 39, Code 93 and 93i, EAN/JAN-13, EAN/JAN 8, IATA Code 2 of 5, Interleaved 2 of 5, Matrix 2 of 5, MSI, GS1 Databar, UPC-A, UPC E, UPC-A/EAN-13 with Extended Coupon Code, Coupon GS1 Code 32(PARAF), EAN-UCC Emulation, GS1 Data Bar 2D Stacked: Codablock A, Codablock F, PDF417, MicroPDF417 2D Matrix: Aztec Code, Data Matrix, MaxiCode, QR Code, Chinese Sensible (Han Xin), Grid Matrix, Dot Code Postal: Australian Post, British Post, Canadian Post, China Post, Japanese Post, Korea Post, Netherlands Post, Planet Code, Postnet |

3) RFID(HF/NFC)

| RFID | HF |
| --- | --- |
| Protocol | ISO15693、ISO14443 A/B 、Mifare、Felica |
| Frequency | 13.56MHz |
| Mode | Card Emulation、Peer-to-Peer、Reader via Host and UICC |
| Reading Range | Read distance 0-6cm (ISO15693), 0-4cm(ISO14443A/B) |

Network Connections

1) Positioning

| Positioning | Specification |
| --- | --- |
| Mode | GPS、A-GPS、BEIDOU、GLONASS、Galileo |
| Frequency | GPS（ L1 1.575GHz C/A code）， BEIDOU（ B1 1.561GHz）， GLONASS(L1 1.602GHz)，Galileo（E1 1.589 GHz / E2 1.561 GHz） |
| Cold start time | Less than 40s |
| Max channel | 31 channel |
| sensitivity | -130dB(SNR value 40 dBHz) |
| Precision | 5-10 Meters（open space） |

2) Wireless LAN

| WLAN | Specification |
| --- | --- |
| Protocol | IEEE 802.11a/b/g/n/ac/d/e/h/i/k/r/v/w （2.4G/5G dual band Wi-Fi） |
| Working Channels | CH1~CH13，CH34-CH140，CH149~CH165, depends on country (region) |
| Security and Encryption | 802.1x,TKIP, AES, PEAPv1, EAP-TLS, EAP-TTLS, PEAP-MSCHAPv2, PEAP-TLS, PEAP-GTC, PWD,SIM, AKA, WEP, WPA/WPA2-PSK |
| Fast roaming | PMIKD caching;Support 802.11r fast roaming(Over the-Air), roaming threshold can be set |

3) Wireless WAN

| WWAN | Specification |
| --- | --- |
| Default frequency band | TD-LTE(B34/B38/B39/B40/B41) FDD-LTE(B1/B2/B3/B4/B5/B7/B8/B20/B28A/B28B) WCDMA(B1/B2/B4/B5/B8) TD-SCDMA(B34/B39) CDMA&EVDO:BC0 GSM/EDGE/GPRS(850/900/1800/1900） |

4) Wireless PAN

| BT | Specification |
| --- | --- |
| Mode | BT5.0 + BR/EDR + BLE |
| Range | More than 10 Meters |

Accessories

|  | Type |
| --- | --- |
| Standard Accessory | Adapter*1, Type-C cable*1, Battery*1, Instructions & Security Information & Warranty Card*1 |
| Optional Accessory | Single Slot Charging Cradle, Screen Protector, Hand Strap, Hand Rope, Rugged Boot |